Revision Date: 27-Oct-2014

Safety Data Sheet

Michigan Chemical Products

1. PRODUCT AND COMPANY IDENTIFICATION

# Product Identifier

Product Name:	Grease Eraser

# Product Number:

Recommended Use:	Cleaning agent

Uses Advised Against:	For Industrial and Institutional Use Only

Manufacturer/Supplier:	Michigan Chemical Products

31400 Kendall Ste. B Fraser, Mi 48026 USA

586-415-8891

# 24 Hour Emergency Phone Numbers:

Medical Emergency/Information:

Transportation/Spill/Leak:	CHEMTEL 800-255-3924

2. HAZARDS IDENTIFICATION

# GHS Classification

Skin Corrosion/Irritation:	Category 2 Serious Eye Damage/Eye Irritation:	Category 2A

# GHS Label Elements

Signal Word:	Warning Symbols:	  Hazard Statements:	Causes skin irritation.

Causes serious eye irritation

# Precautionary Statements:

Prevention:	Wash hands and any exposed skin thoroughly after handling.

Wear protective gloves Wear eye / face protection

# Response:

-Eyes	IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical attention.

-Skin	IF ON SKIN (or hair): Wash with plenty of soap and water. If skin irritation occurs: Get medical attention. Take off contaminated clothing and wash before reuse.

-Specific Treatment:	See Safety Data Sheet Section 4: "FIRST AID MEASURES" for additional information.

Storage:	Not Applicable

Disposal:	Not Applicable

# Hazards Not Otherwise Classified:

Not Applicable

Other Information:	• May be harmful if swallowed.

- Harmful if absorbed through skin.

- Inhalation of vapors or mist may cause respiratory irritation.

- Keep out of reach of children.

3. COMPOSITION/INFORMATION ON INGREDIENTS

| Chemical Name | CAS No | Weight-% |
| --- | --- | --- |
| water | 7732-18-5 | 60-100 |
| sodium dodecylbenzene sulfonate | 25155-30-0 | 1-5 |
| 2-butoxyethanol | 111-76-2 | 1-5 |
| diethylene glycol monobutyl ether | 112-34-5 | 1-5 |
| trisodium phosphate | 7601-54-9 | 1-3 |

Specific chemical identity and/or exact percentage of composition has been withheld as a trade secret.

4. FIRST AID MEASURES

-Eye Contact:	Rinse cautiously with water for at least 15 minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical attention.

-Skin Contact:	Wash with plenty of soap and water. Take off contaminated clothing and wash before reuse. If skin irritation occurs: Get medical attention.

-Inhalation:	Remove victim to fresh air and keep at rest in a position comfortable for breathing. Call a poison control center or physician if you feel unwell.

-Ingestion:	Rinse mouth. Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Get medical attention if you feel unwell.

Note to Physicians:	Treat symptomatically.

5. FIRE-FIGHTING MEASURES

Suitable Extinguishing Media:	Product does not support combustion. Use extinguishing agent suitable for type of

surrounding fire

# Specific Hazards Arising from the Chemical:

Dried product is capable of burning. Combustion products are toxic.

Hazardous Combustion Products:	May include Carbon monoxide Carbon dioxide and other toxic gases or vapors.

# Protective Equipment and Precautions for Firefighters:

Wear MSHA/NIOSH approved self-contained breathing apparatus (SCBA) and full protective gear. Cool fire-exposed containers with water spray.

6. ACCIDENTAL RELEASE MEASURES

Personal Precautions:	Avoid contact with skin, eyes or clothing. Use personal protective equipment as required.

Environmental Precautions:	Do not rinse spill onto the ground, into storm sewers or bodies of water.

Methods for Clean-Up:	Prevent further leakage or spillage if safe to do so. Contain and collect spillage with

non-combustible absorbent material, (e.g. sand, earth, diatomaceous earth, vermiculite) and place in container for disposal according to local / national regulations (see Section 13).

7. HANDLING AND STORAGE

# Advice on Safe Handling:

Handle in accordance with good industrial hygiene and safety practice. Wash thoroughly after handling.

Storage Conditions:	Keep containers tightly closed in a dry, cool and well-ventilated place. Keep out of the reach of  children. Keep from freezing.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

# Occupational Exposure Limits:	.

| Chemical Name | ACGIH TLV | OSHA PEL | NIOSH |
| --- | --- | --- | --- |
| 2-butoxyethanol 111-76-2 | TWA: 20 ppm | TWA: 50 ppm TWA: 240 mg/m3 (vacated) TWA: 25 ppm (vacated) TWA: 120 mg/m3 (vacated) S* S* | IDLH: 700 ppm TWA: 5 ppm TWA: 24 mg/m3 |

Engineering Controls:	Provide good general ventilation.

If work practices generate dust, fumes, gas, vapors or mists which expose workers to chemicals above the occupational exposure limits, local exhaust ventilation or other engineering controls should be considered.

# Personal Protective Equipment

Eye/Face Protection:	Wear splash goggles.

Skin and Body Protection:	Wear rubber or other chemical-resistant gloves.

Respiratory Protection:	Not required with expected use.

If occupational exposure limits are exceeded or respiratory irritation occurs, use of a NIOSH approved respirator suitable for the use-conditions and chemicals in Section 3 should be considered.

General Hygiene Considerations:	Wash hands and any exposed skin thoroughly after handling.

See 29 CFR 1910.132-138 for further guidance.

9. PHYSICAL AND CHEMICAL PROPERTIES

| Appearance/Physical State: | Liquid |
| --- | --- |
| Color: | Clear |
| Odor: | Mild |
| pH: | < 11% |
| Melting Point / Freezing Point: | No information available. |
| Boiling Point / Boiling Range: | 100 °C / 212 °F |
| Flash Point: | > 100 °C / > 212 °F |
| Evaporation Rate: | < 1 |
| Flammability (solid, gas) | No information available. |
| Upper Flammability Limit: | No information available. |
| Lower Flammability Limit: | No information available. |
| Vapor Pressure: | No information available. |
| Vapor Density: | No information available. |
| Specific Gravity: | 1.04 |
| Solubility(ies): | Soluble in water |
| Partition Coefficient: | No information available. |
| Autoignition Temperature: | No information available. |
| Decomposition Temperature: | No information available. |
| Viscosity: | No information available. |

10. STABILITY AND REACTIVITY

Reactivity:	This material is considered to be non-reactive under normal conditions of use.

Chemical Stability:	Stable under normal conditions.

Possibility of Hazardous Reactions: Not expected to occur with normal handling and storage.

Conditions to Avoid:	Extremes of temperature and direct sunlight.

Incompatible Materials:	Strong oxidizing agents. Strong acids.

# Hazardous Decomposition Products:

May include carbon monoxide, carbon dioxide (CO2) and other toxic gases or vapors.

11. TOXICOLOGICAL INFORMATION

Likely Routes of Exposure:	Eyes, Skin, Ingestion, Inhalation.

# Symptoms of Exposure:

-Eye Contact:	Pain, redness, swelling of the conjunctiva and blurred vision.

-Skin Contact:	Pain, redness and cracking of the skin.

-Inhalation:	Nasal discomfort and coughing.

-Ingestion:	Pain, nausea, vomiting and diarrhea.

# Immediate, Delayed, Chronic Effects

Product Information:	Data not available or insufficient for classification.

Target Organ Effects:	-Eyes. Respiratory System. -Skin. Blood. Central nervous system. hematopoietic system.

kidney. Liver.

# Numerical Measures of Toxicity

The following acute toxicity estimates (ATE) are calculated based on the GHS document.

.

ATEmix (oral):	6195 mg/kg

ATEmix (dermal):	13585 mg/kg ATEmix (inhalation-dust/mist):	29.7 mg/l ATEmix (inhalation-vapor):	74.2 mg/l

# Component Acute Toxicity Information

| Chemical Name | Oral LD50 | Dermal LD50 | Inhalation LC50 |
| --- | --- | --- | --- |
| water 7732-18-5 | > 90 mL/kg ( Rat ) | Not Available | Not Available |
| sodium dodecylbenzene sulfonate 25155-30-0 | = 438 mg/kg ( Rat ) | Not Available | Not Available |
| 2-butoxyethanol 111-76-2 | = 470 mg/kg ( Rat ) | = 2270 mg/kg ( Rat ) = 220 mg/kg ( Rabbit ) | = 2.21 mg/L ( Rat ) 4 h = 450 ppm ( Rat ) 4 h |
| diethylene glycol monobutyl ether 112-34-5 | = 3384 mg/kg ( Rat ) | = 2700 mg/kg ( Rabbit ) | Not Available |
| sodium tripolyphosphate 7758-29-4 | = 3100 mg/kg ( Rat ) | > 7940 mg/kg ( Rabbit ) | Not Available |
| trisodium phosphate 7601-54-9 | > 2000 mg/kg ( Rat ) | > 2 mg/kg ( Rabbit ) | > 2.16 mg/L ( Rat ) 1 h |

Carcinogenicity: No components present at 0.1% or greater are listed as to being carcinogens by ACGIH, IARC, NTP or OSHA.

12. ECOLOGICAL INFORMATION

# Ecotoxicity

| Chemical Name | Algae/Aquatic Plants | Fish | Toxicity to Microorganisms | Crustacea |
| --- | --- | --- | --- | --- |
| sodium dodecylbenzene sulfonate 25155-30-0 | Not Available | 10.8: 96 h Oncorhynchus mykiss mg/L LC50 static | Not Available | Not Available |
| 2-butoxyethanol 111-76-2 | Not Available | 1490: 96 h Lepomis macrochirus mg/L LC50 static 2950: 96 h Lepomis macrochirus mg/L LC50 | Not Available | 1698 - 1940: 24 h Daphnia magna mg/L EC50 1000: 48 h Daphnia magna mg/L EC50 |
| diethylene glycol monobutyl ether 112-34-5 | 100: 96 h Desmodesmus subspicatus mg/L EC50 | 1300: 96 h Lepomis macrochirus mg/L LC50 static | Not Available | 2850: 24 h Daphnia magna mg/L EC50 100: 48 h Daphnia magna mg/L EC50 |
| sodium tripolyphosphate 7758-29-4 | Not Available | 1650: 48 h Leuciscus idus mg/L LC50 | Not Available | Not Available |

Persistence and Degradability:	No information available.

Bioaccumulation:	No information available.

Other Adverse Effects:	No information available.

13. DISPOSAL CONSIDERATIONS

Disposal of Wastes:	Dispose of in accordance with federal, state and local regulations.

Contaminated Packaging:	Dispose of in accordance with federal, state and local regulations.

14. TRANSPORT INFORMATION

DOT:	Not Regulated

Proper Shipping Name:	Non-Hazardous Product

15. REGULATORY INFORMATION

TSCA Status: (Toxic Substance Control Act Section 8(b) Inventory)

All chemical substances in this product are included on or exempted from listing on the TSCA Inventory of Chemical Substances.

# SARA 313

This product contains the following listed substances:

# 2-butoxyethanol

CAS No 111-76-2 applies to R-(OCH2CH2)n-OR', where n = 1, 2, or 3, R=Alkyl C7 or less, or R = Phenyl or Alkyl substituted phenyl, R' = H or Alkyl C7 or less, or OR' consisting of Carboxylic acid ester, Sulfate, Phosphate, Nitrate, or Sulfonate, Chemical Category N230

# diethylene glycol monobutyl ether

CAS No 112-34-5 applies to R-(OCH2CH2)n-OR', where n = 1, 2, or 3, R=Alkyl C7 or less, or R = Phenyl or Alkyl substituted phenyl, R' = H or Alkyl C7 or less, or OR' consisting of Carboxylic acid ester, Sulfate, Phosphate, Nitrate, or Sulfonate, Chemical Category N230

# SARA 311/312 Hazard Categories

Acute Health Hazard:	Yes

Chronic Health Hazard:	Yes

Fire Hazard:	No

Sudden release of pressure hazard:	No

Reactive Hazard:	No

California Proposition 65

This product contains the following Proposition 65 chemicals

| Chemical Name | California Proposition 65 |
| --- | --- |
| diethanolamine - 111-42-2 | Carcinogen |
| basic violet 10 - 81-88-9 | Carcinogen |

WARNING: This product contains a chemical known to the State of California to cause cancer.

16. OTHER INFORMATION

# NFPA	Health Hazards: 2

Flammability: 0	Instability: 0	Special: N/A

HMIS

Health Hazards: 2*	Flammability: 0	Physical Hazards: 0

Revision Date:	27-Oct-2014

Reasons for Revision:	No information available.

# Disclaimer:

The information provided in this Material Safety Data Sheet is correct to the best of our knowledge, information and belief at the date of its publication. The information given is designed only as a guidance for safe handling, use, processing, storage, transportation, disposal and release and is not to be considered a warranty or quality specification. The information relates only to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.

End of Safety Data Sheet